Analyzing and detecting the original signal flow in paralyzed patients involves intercepting motor intentions at various points in the nervous system—primarily the brain, peripheral nerves, or residual muscle activity—using brain-computer interfaces (BCIs) and machine learning (ML). These systems decode neural activity (motor intention) to restore communication or movement, even when the connection between the brain and muscles is severed.

Here is how to analyze and detect these signals:

1. Signal Acquisition Modalities

- Electroencephalography (EEG): Non-invasive, scalp-attached electrodes measure electrical changes (e.g., motor imagery). It is used to detect movement intent, such as closing a hand.

- Electrocorticography (ECoG): Electrodes are placed on the brain's surface, providing higher spatial resolution and spectral bandwidth than EEG.

- Intracortical Microelectrode Arrays: Invasive electrodes penetrate the motor cortex to record single-neuron firing rates (action potentials), providing the highest fidelity for movement kinematics.

- Electromyography (EMG): Surface or needle electrodes detect electrical activity in muscles, suitable for detecting attempts at movement in partially paralyzed muscles.

- Peripheral Nerve Recordings: Intrafascicular electrodes can be implanted in main nerves (e.g., radial, ulnar) to detect motor signals directly.

2. Analysis and Feature Extraction Techniques

Once signals are recorded, they must be processed to identify relevant movement intentions:

- Event-Related Desynchronization (ERD) & Synchronization (ERS): EEG analysis looks for a decrease (desynchronization) in

(14–30 Hz) bands during movement intention, followed by an increase (synchronization) upon completion.

- Motor-Related Cortical Potentials (MRCP): A slow negative potential observed in EEG (~<1 Hz) starting up to 2 seconds before voluntary movement.

- Autocorrelation Analysis: Analyzes the time domain of EEG, finding that the autocorrelation decays slower during movement intention compared to rest.

- Spiking-Band Power (SBP): In invasive setups, high-pass filtered (300–1000 Hz) signals are used to capture neuron firing intensity.

- Waveform Length (WL): For EMG signals, this combines amplitude and frequency information to determine muscle activity.

3. Detection and Classification of Movement

Machine learning algorithms are used to classify extracted features into distinct actions:

- Linear Discriminant Analysis (LDA): Used to classify EEG/EMG signals into resting or movement attempt states.

- Support Vector Machines (SVM): Frequently used for classifying EMG patterns of specific movements (e.g., hand open/close).

- Kalman Filters/Neural Networks: Employed for continuous decoding of movement trajectories (velocity, direction) from intracortical signals.

- SynergyNet: A deep learning framework used to decode complex hand movements by learning the underlying neural-motor synergies rather than individual joints.

4. Key Considerations

- Ipsilesional vs. Contralesional: For stroke patients, recording from the ipsilesional side (damaged hemisphere) is crucial for rehabilitation, though contralesional signals can sometimes be used.

- Artifact Removal: Crucial to remove non-neural signals, such as eye blinks (EOG) or muscle activity (EMG) from EEG, using techniques like Independent Component Analysis (ICA).

- Hybrid Systems: Combining EEG (for high-level intent) and EMG (for low-level muscle activity) provides better accuracy than either alone.

- Signal Drift: Neural signals change over time; adaptive classifiers are required to recalibrate the model.

For maximum accuracy, a hybrid BCI system (combining EEG and EMG) with Machine Learning classifiers (LDA/SVM) can achieve 90%+ accuracy in detecting voluntary movement attempts.